#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

#include <string.h>

#include "sdcard.h"

SD_Context sd_ctx;

FAT32_Info fat_fs;

void my_delay_ms(uint32_t ms) {

while (ms--) _delay_ms(1);

}

/* --- UART --- */

#define BAUD 9600

#define MYUBRR (F_CPU/16/BAUD-1)

void uart_init(uint16_t ubrr) {

UBRR0H = (uint8_t)(ubrr >> 8);

UBRR0L = (uint8_t)ubrr;

UCSR0B = (1 << TXEN0);

UCSR0C = (1 << UCSZ01) | (1 << UCSZ00);

}

int usart_putchar(char c, FILE *stream) {

if (c == '\n') usart_putchar('\r', stream);

while (!(UCSR0A & (1 << UDRE0)));

UDR0 = c;

return 0;

}

static FILE mystdout = FDEV_SETUP_STREAM(usart_putchar, NULL, _FDEV_SETUP_WRITE);

/* --- CALLBACKS DE TESTE --- */

// T6: Callback de Append (Escreve 2 blocos de 512 bytes: 'A's e 'B's)

uint16_t my_stream_reader(const uint8_t **data) {

static uint8_t counter = 0;

if (counter >= 2) {

counter = 0; // Reseta para o próximo teste

return 0;

}

// Preenche o buffer da biblioteca para economizar RAM

memset(sd_ctx.buffer, 'A' + counter, 512);

*data = sd_ctx.buffer;

counter++;

return 512;

}

// T8: Callback de Leitura (Exibe os primeiros 16 bytes de cada bloco lido)

void my_stream_callback(const uint8_t *buffer, uint16_t len) {

static uint16_t total_lido = 0;

printf("Lido bloco de %u bytes. Inicio: ", len);

for(uint8_t i=0; i<16; i++) putchar(buffer[i]);

printf("...\n");

total_lido += len;

}

/* Variável global para sinalizar o fim do disco ao callback */

volatile uint8_t disk_is_full = 0;

/* Callback de Estresse: Entrega 512 bytes sem parar até disk_is_full ser 1 */

uint16_t stress_test_reader(const uint8_t **data) {

if (disk_is_full) return 0; // Para o streaming quando o disco encher

memset(sd_ctx.buffer, 'W', 512);

*data = sd_ctx.buffer;

return 512;

}

int main(void) {

uart_init(MYUBRR);

stdout = &mystdout;

_delay_ms(1000);

printf("\n=== TESTE DE CAPACIDADE MAXIMA ===\n");

sd_ctx.hal.delay_ms = my_delay_ms;

sd_begin(&sd_ctx, my_delay_ms);

sd_fat_mount(&sd_ctx, &fat_fs);

SD_FileName file;

sd_set_filename(&file, "FULL.DAT");

sd_file_delete(&sd_ctx, &fat_fs, &file);

sd_file_create(&sd_ctx, &fat_fs, &file);

printf("Escrevendo... (Aguarde o erro DISK_FULL)\n");

/*

Chamamos a função UMA ÚNICA VEZ.

Ela vai rodar o callback infinitamente até encontrar um erro na FAT.

*/

SD_Status status = sd_file_append_stream(&sd_ctx, &fat_fs, &file, stress_test_reader);

if (status == SD_ERR_DISK_FULL) {

printf("\n[SUCESSO] O cartao reportou DISCO CHEIO corretamente!\n");

} else {

printf("\n[STATUS] Retorno da biblioteca: %d\n", status);

}

uint32_t final_size;

sd_file_get_size(&sd_ctx, &fat_fs, &file, &final_size);

printf("Tamanho final do arquivo: %lu bytes\n", final_size);

printf("\n=== FIM DO TESTE ===\n");

while(1);

}