#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

#include <string.h>

#include "sdcard.h"

SD_Context sd_ctx;

FAT32_Info fat_fs;

/* ---------------- UART ---------------- */

#define BAUD 9600

#define MYUBRR (F_CPU/16/BAUD-1)

void uart_init(uint16_t ubrr) {

UBRR0H = (uint8_t)(ubrr >> 8);

UBRR0L = (uint8_t)ubrr;

UCSR0B = (1 << TXEN0);

UCSR0C = (1 << UCSZ01) | (1 << UCSZ00);

}

int usart_putchar(char c, FILE *stream) {

if (c == '\n') usart_putchar('\r', stream);

while (!(UCSR0A & (1 << UDRE0)));

UDR0 = c;

return 0;

}

static FILE mystdout = FDEV_SETUP_STREAM(usart_putchar, NULL, _FDEV_SETUP_WRITE);

/* ---------------- DELAY ---------------- */

void my_delay_ms(uint32_t ms) {

while (ms--) _delay_ms(1);

}

/* ========================================================= */

/* CALLBACKS */

/* ========================================================= */

/* APPEND: gera 2 blocos (A e B) */

uint16_t append_cb(const uint8_t **data) {

static uint8_t count = 0;

if (count >= 2) {

count = 0;

return 0;

}

memset(sd_ctx.buffer, 'A' + count, 512);

*data = sd_ctx.buffer;

count++;

return 512;

}

/* WRITE STREAM: sobrescreve com 'X' */

uint16_t write_cb(uint8_t *data, uint16_t max_len) {

memset(data, 'X', max_len);

return max_len;

}

/* READ STREAM */

void read_cb(const uint8_t *buffer, uint16_t len) {

printf("Chunk %u: ", len);

for (uint8_t i = 0; i < 16; i++) putchar(buffer[i]);

printf("\n");

}

/* ========================================================= */

/* TESTES */

/* ========================================================= */

void test_all(void) {

SD_FileName file;

sd_set_filename(&file, "TESTE.TXT");

printf("\n[T4] Criar arquivo\n");

sd_file_delete(&sd_ctx, &fat_fs, &file);

printf("create: %d\n", sd_file_create(&sd_ctx, &fat_fs, &file));

printf("\n[T6] Append (gera clusters)\n");

printf("append: %d\n", sd_file_append_stream(&sd_ctx, &fat_fs, &file, append_cb));

printf("\n[T9] Get size\n");

uint32_t size;

sd_file_get_size(&sd_ctx, &fat_fs, &file, &size);

printf("size: %lu\n", size);

printf("\n[T5] Write stream (overwrite)\n");

printf("write: %d\n", sd_file_write_stream(&sd_ctx, &fat_fs, &file, write_cb));

printf("\n[T7] Read full\n");

uint8_t buffer[64];

sd_file_read(&sd_ctx, &fat_fs, &file, buffer, sizeof(buffer));

for (uint8_t i = 0; i < 32; i++) putchar(buffer[i]);

printf("\n");

printf("\n[T8] Read streaming\n");

sd_file_read_stream(&sd_ctx, &fat_fs, &file, read_cb);

printf("\n[T10] Delete\n");

printf("delete: %d\n", sd_file_delete(&sd_ctx, &fat_fs, &file));

printf("\n[Listagem final]\n");

sd_dir_list(&sd_ctx, &fat_fs);

}

/* ========================================================= */

/* MAIN */

/* ========================================================= */

int main(void) {

uart_init(MYUBRR);

stdout = &mystdout;

_delay_ms(1000);

printf("\n=== TESTES SD CARD ===\n");

sd_begin(&sd_ctx, my_delay_ms);

if (sd_fat_mount(&sd_ctx, &fat_fs) != SD_SUCCESS) {

printf("Erro FAT\n");

while(1);

}

test_all();

printf("\n=== FIM ===\n");

while (1);

}