#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

#include <string.h>

#include "sdcard.h"

SD_Context sd_ctx;

FAT32_Info fat_fs;

#define MB (1024UL * 1024UL)

/* Escolha aqui o intervalo */

#define REPORT_STEP (10 * MB)   // ou 100 * MB

volatile uint32_t total_written = 0;

volatile uint32_t next_report = REPORT_STEP;

/* ---------------- UART ---------------- */

#define BAUD 9600

#define MYUBRR (F_CPU/16/BAUD-1)

void uart_init(uint16_t ubrr) {

UBRR0H = (uint8_t)(ubrr >> 8);

UBRR0L = (uint8_t)ubrr;

UCSR0B = (1 << TXEN0);

UCSR0C = (1 << UCSZ01) | (1 << UCSZ00);

}

int usart_putchar(char c, FILE *stream) {

if (c == '\n') usart_putchar('\r', stream);

while (!(UCSR0A & (1 << UDRE0)));

UDR0 = c;

return 0;

}

static FILE mystdout = FDEV_SETUP_STREAM(usart_putchar, NULL, _FDEV_SETUP_WRITE);

/* ---------------- DELAY ---------------- */

void my_delay_ms(uint32_t ms) {

while (ms--) _delay_ms(1);

}

/* ========================================================= */

/* CALLBACKS */

/* ========================================================= */

/* APPEND: gera 2 blocos (A e B) */

uint16_t append_cb(const uint8_t **data) {

static uint8_t count = 0;

if (count >= 2) {

count = 0;

return 0;

}

memset(sd_ctx.buffer, 'A' + count, 512);

*data = sd_ctx.buffer;

count++;

return 512;

}

/* WRITE STREAM: sobrescreve com 'X' */

uint16_t write_cb(uint8_t *data, uint16_t max_len) {

memset(data, 'X', max_len);

return max_len;

}

/* READ STREAM */

void read_cb(const uint8_t *buffer, uint16_t len) {

printf("Chunk %u: ", len);

for (uint8_t i = 0; i < 16; i++) putchar(buffer[i]);

printf("\n");

}

uint16_t fill_disk_cb(const uint8_t **data)

{

memset(sd_ctx.buffer, 'Z', 512);

*data = sd_ctx.buffer;

total_written += 512;

if (total_written >= next_report)

{

printf("[PROGRESSO] %lu MB escritos\n", total_written / MB);

next_report += REPORT_STEP;

}

return 512;

}

void test_dir_full(void)

{

printf("\n[T11] Diretorio cheio\n");

SD_FileName file;

char name[13];

uint16_t count = 0;

SD_Status st;

while (1)

{

sprintf(name, "F%05u.TXT", count);

sd_set_filename(&file, name);

st = sd_file_create(&sd_ctx, &fat_fs, &file);

if (st != SD_SUCCESS)

{

printf("Parou em %u arquivos\n", count);

printf("Status: %d (esperado: SD_ERR_DISK_FULL)\n", st);

break;

}

count++;

if (count % 50 == 0)

printf("Criados: %u\n", count);

}

}

void test_file_not_found(void)

{

printf("\n[T12] Arquivo inexistente\n");

SD_FileName file;

sd_set_filename(&file, "NOFILE.TXT");

uint8_t buffer[32];

uint32_t size;

SD_Status st;

/* READ */

st = sd_file_read(&sd_ctx, &fat_fs, &file, buffer, sizeof(buffer));

printf("Read status: %d (esperado: %d)\n", st, SD_ERR_NOT_FOUND);

/* WRITE */

st = sd_file_write_stream(&sd_ctx, &fat_fs, &file, write_cb);

printf("Write status: %d (esperado: %d)\n", st, SD_ERR_NOT_FOUND);

/* SIZE */

st = sd_file_get_size(&sd_ctx, &fat_fs, &file, &size);

printf("Size status: %d (esperado: %d)\n", st, SD_ERR_NOT_FOUND);

}

void test_disk_full(void)

{

printf("\n[T13] Enchendo disco com monitoramento...\n");

total_written = 0;

next_report = REPORT_STEP;

SD_FileName file;

sd_set_filename(&file, "FULL2.DAT");

sd_file_delete(&sd_ctx, &fat_fs, &file);

sd_file_create(&sd_ctx, &fat_fs, &file);

SD_Status st = sd_file_append_stream(&sd_ctx, &fat_fs, &file, fill_disk_cb);

if (st == SD_ERR_DISK_FULL)

{

printf("\n[OK] Disco cheio detectado\n");

}

else

{

printf("\n[ERRO] Status: %d\n", st);

}

uint32_t size;

sd_file_get_size(&sd_ctx, &fat_fs, &file, &size);

printf("Total escrito: %lu MB\n", size / MB);

}

/* ========================================================= */

/* TESTES */

/* ========================================================= */

void test_all(void) {

SD_FileName file;

sd_set_filename(&file, "TESTE.TXT");

printf("\n[T4] Criar arquivo\n");

sd_file_delete(&sd_ctx, &fat_fs, &file);

printf("create: %d\n", sd_file_create(&sd_ctx, &fat_fs, &file));

printf("\n[T6] Append (gera clusters)\n");

printf("append: %d\n", sd_file_append_stream(&sd_ctx, &fat_fs, &file, append_cb));

printf("\n[T9] Get size\n");

uint32_t size;

sd_file_get_size(&sd_ctx, &fat_fs, &file, &size);

printf("size: %lu\n", size);

printf("\n[T5] Write stream (overwrite)\n");

printf("write: %d\n", sd_file_write_stream(&sd_ctx, &fat_fs, &file, write_cb));

printf("\n[T7] Read full\n");

uint8_t buffer[64];

sd_file_read(&sd_ctx, &fat_fs, &file, buffer, sizeof(buffer));

for (uint8_t i = 0; i < 32; i++) putchar(buffer[i]);

printf("\n");

printf("\n[T8] Read streaming\n");

sd_file_read_stream(&sd_ctx, &fat_fs, &file, read_cb);

printf("\n[T10] Delete\n");

printf("delete: %d\n", sd_file_delete(&sd_ctx, &fat_fs, &file));

printf("\n[Listagem final]\n");

sd_dir_list(&sd_ctx, &fat_fs);

}

/* ========================================================= */

/* MAIN */

/* ========================================================= */

int main(void) {

uart_init(MYUBRR);

stdout = &mystdout;

_delay_ms(1000);

printf("\n=== INICIANDO TESTES SD CARD ===\n");

// 1. Inicializa Hardware e Monta o Sistema de Arquivos PRIMEIRO

if (sd_begin(&sd_ctx, my_delay_ms) != SD_SUCCESS) {

printf("Erro Hardware SD\n");

while(1);

}

if (sd_fat_mount(&sd_ctx, &fat_fs) != SD_SUCCESS) {

printf("Erro FAT\n");

while(1);

}

// 2. Teste de Data e Hora (Usando a estrutura g_sd_datetime global)

SD_FileName file_aux;

printf("[DATA/HORA] Criando arquivos com tempos diferentes...\n");

g_sd_datetime.hour = 10;

g_sd_datetime.minute = 30;

sd_set_filename(&file_aux, "TIME1030.TXT");

sd_file_create(&sd_ctx, &fat_fs, &file_aux);

g_sd_datetime.hour = 15;

g_sd_datetime.minute = 45;

sd_set_filename(&file_aux, "TIME1545.TXT");

sd_file_create(&sd_ctx, &fat_fs, &file_aux);

// 3. Executa a bateria de testes padrão

test_all();

// 4. Teste de performance e limites (Onde a otimização de cluster brilha)

test_dir_full();

test_file_not_found();

printf("\n=== FIM DE TODOS OS TESTES ===\n");

while (1);

}